# Predicting the Mechanical Properties of Complex Lattice Metamaterials Using Graph Neural Networks

Alexander Adams1, Xing Liu2 (Advisor)

1Department of Mechanical Engineering and Materials Science, University of Pittsburgh, Pittsburgh, PA 15260

2Department of Mechanical and Industrial Engineering, New Jersey Institute of Technology, Newark, NJ 07102

Abstract: Lattice metamaterials are engineered solids whose performance is governed not only by constituent material properties, but also by their geometric architecture. Despite their promise as lightweight, strong, and multifunctional materials, uncertainties in their mechanical properties, including stiffness and fracture resistance, raises concerns about their reliability in practical applications. Such uncertainties arise from imperfections and randomness in the dimensions and spatial arrangements of structs introduced during additive manufacturing. These effects are beyond the capability of classical analytical models, which typically assume idealized, perfectly periodic unit cells. Finite element (FE) simulations can resolve these complexities, but they become computationally prohibitive when used to explore the vast space of possible topologies and defect configurations. This work addresses this challenge by developing a fast, topology-aware surrogate model that frames the prediction of stiffness and fracture resistance as a graph learning problem. A lattice can be naturally represented as a graph, where nodes denote joints, edges denote struts, and features encode geometry and boundary conditions. Graph Neural Networks (GNNs) operate on this representation and propagate information along connected edges in a way that mirrors physical load-path transmission through the lattice, thereby enabling mechanical property prediction. The research methodology proceeds in two phases. First, a synthetic dataset is generated using a custom Abaqus scripting pipeline that produces randomized Voronoi-like beam-lattice geometries. Second, a Graph Convolutional Network (GCN) is trained on this dataset, using node coordinates and graph connectivity as inputs to predict global lattice stiffness. Longer-term goals include extending the model to predict fracture behavior of random and imperfect lattice materials, validating its prediction against Fleck-Deshpande-Ashby scaling laws, and testing generalization to unseen topology families. This approach has the potential to reduce the cost of lattice design iterations by orders of magnitude relative to full FE simulations, enabling defect-aware optimization of metamaterials for aerospace, biomedical, and energy absorption applications.