Voronoi Lattice Simulation

Code Walkthrough & Workflow Reference

Vornoi_Lattice_w_Data___1_.py

# 0. High-Level Overview

This script is an Abaqus scripting-interface program that automates the generation, meshing, simulation, and data export of randomised 2-D Voronoi-like beam-lattice structures. It is designed to produce a large dataset of (geometry, stiffness, stress) tuples for machine-learning research — in this case training a GNN to predict lattice stiffness from topology.

The script runs a sweep of 3,000 simulations, each at a different randomness level (0.0 → 0.5). For each simulation the pipeline is:

- Generate a perturbed hexagonal seed-point grid

- Connect seeds into beam segments (proximity + random degree)

- Remove crossing beams; thin the side walls; fix corner connectivity

- Build an Abaqus model (part, material, section, mesh)

- Apply uniaxial displacement BCs and run a static FE job

- Post-process the ODB to extract stiffness and element stresses

- Write node_features.csv, adjacency_area.csv, lattice_stiffness.csv, stress_matrix.csv to a per-simulation folder

# 1. Imports

from abaqus import *

from abaqusConstants import *

import regionToolset, mesh, math

import job, random, csv, os

These imports fall into three groups:

- Abaqus scripting API (abaqus, abaqusConstants, regionToolset, mesh, job) — these are only available inside the Abaqus Python interpreter. They expose the model database (mdb), assembly, part, step, BC, and job objects used throughout the script.

- Standard Python math & random — used for geometry calculations and stochastic perturbations.

- csv and os — for folder creation and writing the output CSV files.

Because the Abaqus Python environment is Python 2.7-based, the script uses % string formatting (not f-strings) and avoids Python 3 features.

# 2. User Parameters

All tunable knobs live here. Changing these is the primary way to control what lattices are generated and how they are simulated.

## 2a. Grid Geometry

| Variable / Setting | Default Value | Effect on Output |
| --- | --- | --- |
| cell_size | 20.0 | Sets the horizontal spacing between hexagonal seed columns (in mm). Larger values produce a coarser lattice with wider cells. Directly scales the geometry — doubling cell_size doubles all physical dimensions. |
| cell_height | sqrt(3)*cell_size | Derived quantity. Vertical row spacing for a regular hexagonal tiling. Not directly set by the user; it adjusts automatically with cell_size. |
| n | 11 | Grid density — number of seed columns AND rows. An n=11 grid creates up to 11×11 = 121 seed points before boundary trimming. Increase for higher-resolution, more complex lattices; decrease to speed up iteration. |

## 2b. Randomness Sweep

| Variable / Setting | Default Value | Effect on Output |
| --- | --- | --- |
| randomness_start | 0.0 | Randomness value for the first simulation. At 0.0 the seed grid is a perfect hexagonal array — maximally regular topology. |
| randomness_end | 0.5 | Randomness value for the last simulation. At 0.5 each interior seed point can be displaced up to 50% of cell_size in x and y before Voronoi tessellation. Higher values create more disordered, organic-looking lattices. |
| randomness_count | 3000 | Total number of simulations. The randomness value is linearly interpolated between start and end across this many steps, giving one simulation per incremental randomness value. This is the primary dataset size knob. |

The sweep formula is: randomness = randomness_start + (randomness_end - randomness_start) * sim_idx / (randomness_count - 1). Each sim_idx maps to a unique randomness value, but because beam connections are also stochastic, two sims at the same randomness value would still differ.

## 2c. Material Properties

| Variable / Setting | Default Value | Effect on Output |
| --- | --- | --- |
| youngs_modulus | 2000.0 MPa | Elastic stiffness of the beam material. Higher values make all beams stiffer, scaling the overall lattice stiffness proportionally. For a purely linear analysis, k_lattice scales linearly with this value. |
| poissons_ratio | 0.30 | Lateral contraction ratio. Affects in-plane deformation coupling but has relatively minor influence on uniaxial stiffness for slender beams. Typical structural value. |

## 2d. Cross-Section & Mesh

| Variable / Setting | Default Value | Effect on Output |
| --- | --- | --- |
| rect_a | 0.01 mm | Out-of-plane thickness of the rectangular beam cross-section. Very thin out-of-plane (0.01 mm) makes this effectively a 2-D planar problem — moment of inertia about the out-of-plane axis is negligible. Do not increase unless modelling 3-D effects. |
| rect_b | 0.10 mm | In-plane height of the rectangular beam cross-section. This is the primary cross-section size driver. Increasing rect_b raises beam bending stiffness as b^3, strongly affecting k_lattice. |
| seed_size | 0.30 mm | Target mesh element size along beam edges. Smaller values = finer mesh = higher accuracy but longer solve time. The mesh convergence sweet spot for slender beams is typically 2–5 elements per beam. |

## 2e. Loading

| Variable / Setting | Default Value | Effect on Output |
| --- | --- | --- |
| displacement_magnitude | 5.0 mm | Applied vertical displacement on the top row of nodes. The lattice stiffness is back-calculated as k = total_reaction_force / displacement_magnitude. A larger displacement tests nonlinear effects (though nlgeom=OFF here keeps the analysis linear). |
| time_period | 1.0 s | Total pseudo-time for the static loading step. Abaqus uses this for convergence incrementation. For a linear static analysis the physical value is arbitrary; what matters is the ratio of increments. |

Summary:

Code in the user params section essentially determines your synthetic specimen test setup, from features of the lattice such as size to material properties, to the randomness of the spatial variation of the generated seeds, to the magnitude of the simulated loading

# 3. Helper Functions

Four small utility functions are defined before the main loop. They handle low-level geometry tasks used repeatedly during segment generation and overlap removal.

## r6(p)

def r6(p): return (round(p[0], 6), round(p[1], 6))

Rounds both coordinates of a 2-D point to 6 decimal places. This creates a canonical key for floating-point coordinates, preventing false mismatches caused by tiny rounding differences when comparing node positions. Used everywhere a point is used as a dictionary key or set member.

## seg_key(p, q)

def seg_key(p, q): return (r6(p), r6(q)) if r6(p) <= r6(q) else (r6(q), r6(p))

Produces an ordered, canonical key for a beam segment (p, q). By always putting the lexicographically smaller endpoint first, the same physical segment (A→B or B→A) always maps to the same key, so the segments dictionary never stores duplicates.

## distance(p1, p2)

def distance(p1, p2): return math.sqrt((p1[0]-p2[0])**2 + (p1[1]-p2[1])**2)

Standard 2-D Euclidean distance. Used to filter candidate neighbours in the connection phase (only seeds within connection_threshold = 2.5 * cell_size are considered) and to decide which crossing beam to remove (the longer one is removed).

## ccw(a, b, c) & segments_intersect(p1, q1, p2, q2)

def ccw(a, b, c): return (c[1]-a[1])*(b[0]-a[0]) > (b[1]-a[1])*(c[0]-a[0])

def segments_intersect(p1, q1, p2, q2): ...

Classic computational-geometry pair. ccw tests whether three points are in counter-clockwise order using the signed cross-product. segments_intersect uses two ccw tests to determine whether line segments (p1, q1) and (p2, q2) properly cross each other — endpoints touching are explicitly excluded (return False), so T-junctions are allowed.

This is the standard O(1) segment-intersection test from de Berg et al. Properly handles all degenerate orientations except collinear overlap.

## is_boundary_point(pt, y_bottom_fixed, y_top_fixed)

def is_boundary_point(pt, y_bottom_fixed, y_top_fixed): return abs(pt[1] - y_bottom_fixed) < 1e-6 or abs(pt[1] - y_top_fixed) < 1e-6

Returns True if a point sits on the top or bottom boundary row (within floating-point tolerance). Used during overlap removal to protect boundary-spanning beams: if two segments cross and one is a boundary segment, the non-boundary one is removed instead of the boundary one.

Summary:

These functions serve to correct or normalize/standardize the generated geometry features for testing by doing things like truncating coordinates to 6 decimals, using Euclidian distance measuring to determine neighbor filtering, and detecting crossing beams

# 4. Main Simulation Loop

Everything from line 66 onward runs inside for sim_idx in range(randomness_count). Each iteration is one complete simulation: geometry → FEA → export. The loop is broken into sub-sections below.

## 4a. Randomness Calculation & Folder Setup

randomness = randomness_start + (randomness_end - randomness_start) * sim_idx / float(randomness_count - 1)

folder_name = 'randomness_%.4f' % randomness

os.makedirs(folder_path)

The current randomness value is computed by linear interpolation. A dedicated output folder is created for each simulation (e.g. randomness_0.2500/). All four CSV output files for that simulation are written into this folder. The script saves and restores os.getcwd() around each folder change so successive iterations do not break relative paths.

## 4b. Seed Point Generation

The seed points are the centres of the Voronoi cells — i.e. the joints of the lattice prior to tessellation. They are placed on a hexagonal grid and optionally jittered.

Base hexagonal grid:

- Column i has x-offset i * 1.5 * cell_size

- Odd columns are offset vertically by cell_height / 2 (the classic hex stagger)

- Row j within a column has y-offset j * cell_height

Randomness jitter:

- If randomness > 0, each interior seed is displaced by up to randomness * cell_size in both x and y using random.uniform

- Edge seeds (i==0, i==n-1, j==0, j==n-1) are NOT jittered in the perpendicular direction — this keeps the boundary rows and columns perfectly straight, which is essential for applying uniform BCs

Boundary tagging:

- Seeds are classified as bottom_points, top_points, or interior_points based on proximity to y_min / y_max within a tolerance of 0.3 * cell_height

- Bottom and top rows are snapped to exact y_bottom_fixed and y_top_fixed values — again, to ensure clean BC application

The distinction between boundary and interior seeds is critical downstream: it determines which beams are protected during overlap removal and which nodes receive displacement BCs.

## 4c. Build Segments (Proximity + Random Degree)

connection_threshold = cell_size * 2.5

This section creates the beam connectivity. For each seed point:

- Find all other seeds within connection_threshold (2.5 * cell_size)

- Sort by distance; take up to 12 nearest as candidates

- Randomly shuffle those candidates, then pick a random number of connections (3–6)

- Add a canonical seg_key for each chosen connection

The result is stored in segments_dict (a dict keyed by seg_key, which automatically deduplicates). This stochastic connection step is what generates structural variety within each randomness level — even two sims with identical randomness produce different topologies.

The 3–6 connection range approximately replicates Voronoi edge valences (most Voronoi vertices have degree 3; higher degrees add disorder). The 12-candidate cap prevents very long-range connections that would create unrealistic lattices.

## 4d. Remove Overlapping Beams

while changed:

if segments_intersect(p1, q1, p2, q2): ...

Iterates through all pairs of segments looking for proper crossings. On finding one, it removes one of the two crossing beams according to this priority:

- If one beam is a boundary beam (both endpoints on top/bottom row) and the other is not, remove the non-boundary beam

- Otherwise remove the longer of the two crossing beams

The while changed loop repeats until no crossings remain. This can take multiple passes because removing one beam may expose a previously masked crossing. This is an O(n^2) process per iteration, which is acceptable given the relatively small segment counts (typically hundreds, not thousands).

## 4e. Thin Side Walls

side_keep_prob = 0.45

Beams where both endpoints lie within cell_size of the left or right boundary are randomly removed with 55% probability (keep probability 0.45). This simulates the open-face boundary condition typical of physical Voronoi specimens — the side walls of a real foam or lattice test coupon are open, not fully connected. Without this thinning the sides would be over-constrained and produce unrealistically high stiffness.

## 4f. Fix Corner Nodes

Ensures the bottom-left and bottom-right corner nodes each have at least two connections. If a corner node has degree <= 1 (isolated or dangling), the script finds the nearest node above it within a horizontal tolerance of 0.7 * cell_size and adds a beam to connect them. This prevents degenerate BC scenarios where the pinned corner is floating.

## 4g. Final Deduplication

segments = {}

for (p, q) in segment_list: key = seg_key(p, q); segments[key] = (p, q)

All remaining segments are stored in a canonical dictionary, keyed by seg_key, removing any residual duplicates introduced during corner-fixing. This segments dict is the authoritative beam set used for everything downstream.

Summary:

Randomness calculated using interpolation, value is then passed to seed point generation which makes the seed points, generated points passed to connectivity function to connect them, then rest of section is essentially normalizing/cleaning up generation (no overlaps, thin side walls, corner node fixes, then after that all segments are itemized in a dictionary and result is saved for said iteration, and loops for however many iters specified in user params

# 5. Abaqus Model Construction

This section translates the Python-level beam geometry into an Abaqus finite-element model. Each sub-step corresponds to a standard Abaqus modelling workflow.

## 5a. Part & Sketch

sk = model.ConstrainedSketch(name='__profile__', sheetSize=...)

for (p, q) in segments.values(): sk.Line(point1=p, point2=q)

part = model.Part(name='VoronoiPart', dimensionality=TWO_D_PLANAR, type=DEFORMABLE_BODY)

part.BaseWire(sketch=sk)

Draws each beam as a Line in a 2-D planar constrained sketch, then extrudes that sketch into a wire part. A wire part is the correct Abaqus type for beam elements — it has no area, just a 1-D topology that beam elements are assigned to.

## 5b. Material

model.Material(name='LatticeMaterial')

model.materials['LatticeMaterial'].Elastic(table=((youngs_modulus, poissons_ratio),))

Defines a linear isotropic elastic material with the user-specified Young's modulus and Poisson's ratio. This is the only material property set — no plasticity, no damage, consistent with a linear static analysis.

## 5c. Cross-Section Profile & Beam Section

model.RectangularProfile(name='WireProfile', a=rect_a, b=rect_b)

model.BeamSection(name='LatticeSection', integration=DURING_ANALYSIS, profile='WireProfile', material='LatticeMaterial')

Creates a rectangular cross-section profile (rect_a × rect_b) and assigns it as a beam section. DURING_ANALYSIS means Abaqus integrates the section stiffness numerically at solve time rather than pre-computing it, allowing for nonlinear extensions if needed.

The beam orientation is set via N1_COSINES with n1=(0,0,1), meaning the beam's local 1-axis (strong axis) points out of the plane. This is the standard orientation for a 2-D planar beam lattice.

## 5d. Mesh

part.seedPart(size=seed_size, deviationFactor=0.1, minSizeFactor=0.1)

part.generateMesh()

Seeds all wire edges at the target element size (seed_size = 0.30 mm) and generates the mesh. For wire parts, Abaqus automatically uses B21 (2-node linear Timoshenko beam) or similar 1-D beam elements. The mesh density directly controls the accuracy of stress recovery along each beam.

## 5e. Assembly

inst = a.Instance(name='VoronoiInstance', part=part, dependent=ON)

Instantiates the wire part into the model assembly. dependent=ON means the instance shares the part's mesh rather than having an independent copy — standard practice for single-part models.

## 5f. Analysis Step

model.StaticStep(name='LoadStep', previous='Initial', timePeriod=time_period, nlgeom=OFF, ...)

Creates a static general step following the Initial step. nlgeom=OFF keeps it geometrically linear (small-displacement theory). The increment parameters (initialInc=0.1, minInc=1e-5, maxInc=0.5, maxNumInc=1000) control Abaqus's automatic time-stepping; for a linear analysis the solver typically completes in a single increment.

## 5g. Boundary Conditions

Three boundary conditions are applied in total:

- BottomLeftFix (Initial step): fully pins the bottom-left node (u1=0, u2=0, ur3=0). This eliminates rigid-body motion.

- BottomLineU2Fix (Initial step): fixes u2=0 (vertical displacement) for all remaining bottom nodes. Horizontal and rotational DOFs are free, simulating a roller condition.

- TopDisplacement (LoadStep): applies u2 = displacement_magnitude (5 mm upward) to all top nodes. u1 and ur3 are UNSET (free). This is the uniaxial compression/tension load.

The asymmetric bottom BC (one pin + rollers) is a standard statically determinate setup that avoids spurious horizontal reactions while still preventing rigid-body translation and rotation.

## 5h. Field Output

model.fieldOutputRequests['F-Output-1'].setValues(variables=('S', 'U', 'RF', 'COORD', 'SF'))

Requests the following field outputs to be written to the ODB: S (stress), U (displacement), RF (reaction force), COORD (current nodal coordinates), SF (section forces in beams). RF at the top nodes is what determines lattice stiffness in post-processing.

Summary:

This is the code that is used to take everything prior (user imputs and the python generated features) and translate it into functionality for the Abaqus software to actually use it in the FEA solver and then run an incremental analysis on the generated specimen

# 6. Job Submission

mdb.Job(name=job_name, model=model_name, description=...)

mdb.jobs[job_name].submit(consistencyChecking=OFF)

mdb.jobs[job_name].waitForCompletion()

Creates and submits an Abaqus solver job for the current model. waitForCompletion() blocks the Python script until the solver finishes, ensuring post-processing only runs on completed results. consistencyChecking=OFF skips some pre-check steps to speed up throughput for batch runs.

Each job produces a job_name.odb output database file in the working directory. Post-processing reads this file in the next section.

Summary:

This is where you take the python-to-Abaqus translated output and submit it to the solver o actually run the force loading test that will give the deformation

# 7. Post-Processing & File Export

After the solver completes, the script opens the ODB and extracts four datasets, writing each to a CSV in the simulation's output folder.

## 7a. Node Features (node_features.csv)

w.writerow(['node_id', 'x', 'y'])

Collects all unique joint coordinates from the final segment set. Sorts them (for deterministic ordering), assigns integer node_ids (0, 1, 2, ...), and writes their (x, y) positions. This is the node feature matrix input to the GNN — the spatial coordinates are the primary per-node features.

## 7b. Adjacency Matrix (adjacency_area.csv)

adj_binary[i][j] = 1  # symmetric

Writes a binary N×N adjacency matrix. Entry [i][j] = 1 if a beam connects node i to node j (0 otherwise). The matrix is symmetric (undirected graph). This is the graph connectivity input to the GNN.

This file is called adjacency_area.csv for historical reasons — earlier versions stored cross-sectional area here. The current version stores only binary connectivity (0/1).

## 7c. Lattice Stiffness (lattice_stiffness.csv)

k_lattice = total_RF2 / float(displacement_magnitude)

Sums the vertical reaction forces (RF2) at all top-row nodes, then divides by the applied displacement to get the effective axial stiffness in N/mm. This is the scalar regression target for the GNN — the quantity the model is trained to predict from topology alone.

## 7d. Stress Matrix (stress_matrix.csv)

s11 = float(v.data[0])  # axial stress component

The most complex export. For each beam (segment), it:

- Computes each FE element's midpoint from the ODB nodal coordinates

- For each beam, uses a bounding-box prefilter then point-to-segment distance to assign FE elements to their parent beam

- Tracks the maximum absolute S11 (axial stress) across all elements in each beam

- Writes an N×N symmetric matrix where entry [i][j] = maximum axial stress in the beam between nodes i and j (0 if no beam exists)

The inner function point_to_segment_distance(px, py, ax, ay, bx, by) computes the perpendicular distance from a point to a finite line segment using parametric projection, clamping the parameter t to [0, 1] to handle points nearest to endpoints.

The bounding-box prefilter (xmin_box, xmax_box, ymin_box, ymax_box) is a performance optimisation — it avoids computing the full distance for every element-beam pair, reducing the O(elements * beams) cost significantly.

Summary:

Once the solver has finished computing the results, they are then extracted, the node features being the unique coordinates of the joint locations and their x-y positions, the adjacency matrix which is how the GNN determines the graph connectivity (1 and 0 matrix of the joins), the lattice stiffness which is the sum of the vertical reaction forces divided by the applied displacements (this is what the GNN needs to predict), and then the stress matrix which preforms computations to determine the stress in each beam

# 8. Cleanup & Loop Continuation

odb.close()

os.chdir(original_dir)

The ODB is closed to release file handles (important for long batch runs). The working directory is restored to the original path before the next loop iteration, ensuring the next simulation's job files and folder are created in the correct location.

After all randomness_count iterations complete, the script prints ALL SIMULATIONS COMPLETE.

Summary:

This is just standard file management for programming, ensures that each iteration of a simulation is its own unique file and does not get overwritten by the next

# 9. Output File Summary

Each simulation produces a folder randomness_XXXX/ containing four files:

| Variable / Setting | Default Value | Effect on Output |
| --- | --- | --- |
| node_features.csv | N rows × 3 cols | node_id, x, y for every joint. GNN node feature matrix. |
| adjacency_area.csv | (N+1) × (N+1) | Binary symmetric adjacency. GNN edge connectivity. Row/col headers are node ids. |
| lattice_stiffness.csv | 1 value | k_lattice in N/mm. Scalar regression label for the GNN. |
| stress_matrix.csv | (N+1) × (N+1) | Max axial stress per beam, symmetric. Optional edge feature or auxiliary label. |

N varies between simulations because randomness and stochastic connectivity produce lattices with different numbers of nodes. Your GNN loader must handle variable-size graphs.

Summary: Final outputs from script start to finish

# 10. Quick Tips for Modification

Common things you might want to change and where:

- Change dataset size: adjust randomness_count (line 24)

- Change lattice resolution: adjust n (line 19) and cell_size (line 17)

- Change material: adjust youngs_modulus and poissons_ratio (lines 26-27)

- Change beam thickness (stiffness tuning): adjust rect_b (line 30)

- Change applied load: adjust displacement_magnitude (line 34)

- Disable side-wall thinning: set side_keep_prob = 1.0 (around line 239)

- Change connectivity density: modify the random.randint(3, 6) range (line 184)

- Run a single fixed randomness: set randomness_count=1 and randomness_start to the desired value

Addn. Notes:

-Connectivity is stochastic and independent of the seed variation

-Stress export involves non trivial element to beam mapping

End of Document